Digital Watchdog® is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and the lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Cerritos, CA with manufacturing facilities in Seoul, Korea, Digital Watchdog® is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- Digital Watchdog®
	16220 Bloomfield Avenue,

- Cerritos, California 90703 USA

- Phone: +1 888 446-3593

- Web: www.digital-watchdog.com

- E-mail: dw-tech@digital-watchdog.com

MEGApix® 5MP BULLET IP CAMERA

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although the adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 29 - Video Surveillance Remote Devices and Sensors

MEGApix® 5MP BULLET IP CAMERA

- GENERAL

  - SUMMARY

    - The section includes a 5MP high definition IP video camera with vandal-resistant IP67 rated, IK10 impact-resistant bullet housing.

    - Product - A high definition IP bullet housing, based on H.265, H.264 and MJPEG compression, capable of dual streaming 30fps at resolutions up to 5MP (2592 × 1944), with 2.8mm and 3.6mm fixed lens options in a weather-resistant bullet housing.

    - Related Requirements

      - 27 15 01.13 – Video Surveillance Communications Conductors and Cables

      - 28 05 03 – Safety and Security Network Communications Equipment

      - 28 05 19 – Storage Appliances for Electronic Safety and Security

      - 28 06 20 – Schedules for Video Surveillance

      - 28 23 00 – Video Management System

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Backlight compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - fps - Frames Per Second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

        - International Electrotechnical Commission (IEC) – Ingress Protection Rating IP67

        - European standard EN 62262 — equivalent to International Electrotechnical Commission (IEC) – Impact Protection Rating IK10

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic datasheets.

      - Manufacturer’s installation and operation manuals.

      - Warranty documentation.

  - QUALIFICATIONS

    - The manufacturer shall have a minimum of five years of experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened and undamaged container with identification labels intact.

    - Store the camera in a temperature environment of -22°F ~ 140°F (-30°C ~ 60°C), protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - The manufacturer shall provide a 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@digital-watchdog.com

    - Models DWC-MB95Wi28T, DWC-MB95Wi36T

Digital Watchdog model differences:

DWC-MB95Wi28T	2.8mm fixed lens option

DWC-MB95Wi36T	3.6mm fixed lens option

    - Alternates: None

  - GENERAL DESCRIPTION

    - The High Definition IP Video Bullet Camera with embedded video analytics (“IP camera”) shall provide video performance capable of providing selectable resolutions up to 2592 × 1944 pixels at 30 frames per second (fps) and contained within an IP67, IK10-rated bullet housing.

    - The IP camera shall possess the following characteristics:

      - H.265, H.264 and MJPEG compression

      - 5MP

      - Two independent IP video streams (dual streaming)

      - Day/night operation with IR cut filter

      - Built-in IR LEDs providing 164-foot coverage

      - Double-Shutter True Wide Dynamic Range (WDR), 120dB

      - 4 privacy masking zones

      - Integral motion detection

      - 3D digital noise reduction

      - Audio input

      - SD card slot

      - PoE capable

      - Multicast or unicast capable

      - Built-in webserver

      - Dynamic DNS (DDNS) support

      - IP67-rated

      - IK10 impact-resistant rated

  - VIDEO

    - Imager

      - Sensor:	 			5MP 1/2.7" CMOS

      - Minimum illumination

        - Color mode: 		0.02 lux (F1.6 – 30IRE)

        - Black and white mode: 	0 lux (F1.6 – 30IRE)

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Automatic white balance (AWB)

          - Selectable for Auto or Manual

            - Manual mode to provide the ability to configure red offset, blue offset and warmth or coolness of light.

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - Brightness

            - Shutter mode: 	automatic, anti-flickering, or manual – 1/30 to 1/100000

            - Digital slow shutter to decrease the shutter speed in low light

            - Automatic Gain Control (AGC)

            - Backlight compensation (BLC)

            - True Wide Dynamic Range (WDR) level

        - Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

        - Frequency

        - Image sharpness

        - <IR LED control>

        - Chroma

        - Image mirror or flip

        - Digital noise reduction

        - Privacy zone definition, up to 4 zones

      - Lens

        - Lens type: 2.8mm and 3.6mm Fixed Lens Options

Digital Watchdog model differences:

DWC-MB95Wi28T	2.8mm fixed lens

DWC-MB95Wi36T	3.6mm fixed lens

        - Field of view (FOV): 98.5° and 81.4°

Digital Watchdog model differences:

DWC-MB95Wi28T	98.5° horizontal field of view

DWC-MB95Wi36T	81.4° horizontal field of view

        - Double shutter Wide Dynamic Range

        - Shutter speed:  1/30 sec to 1/100000

    - Video Streams

      - The IP camera shall support two configurable video streams, each of which may have the following properties:

        - Compression Type:

          - H.265

          - H.264

          - MJPEG

        - Resolution:

          - 5MP, 4MP, 2K, 3MP, 2.1MP/1080P, 720P (60Hz: 1 ~ 30fps; 50Hz: 1~25fps)

          - HFR mode: 1080P / 720P (60Hz: 1 ~ 60fps; 50Hz: 1~50fps)

        - Frame rate:	0 – 30 fps

    - IR Distance: 	164 feet

    - Motion Detection – The IP camera shall have the ability to detect motion within user-defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - Storage and Recording

      - The IP camera shall have onboard SD card storage.

        - Card Type:	Micro SD/SDHC/SDXC Class 10

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall have the ability to be backed up to alternate media without the removal of the SD card from the camera.

    - ONVIF – Video streams shall be capable of supporting ONVIF protocol, profile S.

    - Other

      - Single Images - The IP camera shall support the .JPG file image screenshot and export.

      - Discovery - The manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Emergency alarm – The IP camera shall allow the sending of a video clip of configurable duration to up to five server locations.

      - Access - The IP camera shall permit up to five users to simultaneously access the camera.

  - AUDIO

    - The IP camera shall provide one-line level input.

    - Compression and Sampling Rate:  G.711 (8 kHz)

  - NETWORK

    - Connectivity: 10/100 BASE-TX Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Secure Sockets Layer (SSL)

      - Network services: Domain Name System (DNS), Network Time Protocol (NTP)

      - Media: Real-Time Streaming Protocol (RTSP)

      - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

    - DDNS – The IP camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The IP camera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings

      - The IP camera shall provide digest authentication, ID and password protection.

      - The IP camera shall provide three levels of user access with password protection.

  - EMBEDDED ANALYTICS

    - Line crossing video analytics.

    - Perimeter intrusion video analytics.

    - Video tampering with scene change detection video analytics.

    - Video tampering with video blur detection video analytics.

    - Video tampering with abnormal color detection video analytics.

  - ADDITIONAL FEATURES

    - System Information

      - The system settings of the IP camera shall be exportable as a separate file.

      - The IP camera shall maintain an access log of the system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The IP camera shall have a built-in web server that supports browser-based configuration using Internet Explorer.

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer a video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion-related

          - emergency

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant record and playback

      - Alerts

        - e-mail setup

        - define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - view and export camera settings

        - view system logs

  - ELECTRICAL

    - Power

      - Sources

        - 12 VDC

        - PoE IEEE 802.3af Class 3. (Adapter not included)

      - Power Consumption

        - <9W

    - Connectors

      - Ethernet: 			RJ-45 connector

      - External power (12 VDC): 	2-conductor power to a terminal block

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 		Die-cast Aluminum

    - Configuration: 		Bullet

    - Environmental Rating: 	IP67

    - IK rating: 			IK10

    - Dimensions (D x H):  	8.69" x 3.16" x 2.77" (220.8 x 80.5 x70.6 mm)

    - Weight:			1.52 lbs (0.69 kg)

    - Temperature:

      - Operating: 		-22°F ~ 140°F (-30°C ~ 60°C)

    - Humidity:			0–95% RH (non–condensing)

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor before camera installation.

  - INSTALLATION

    - The contractor shall follow all Manufacturer published installation procedures and guidelines.

    - Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The IP camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION